GI 00124

GREEN INFRASTRUCTURE SUBMITTAL REQUIREMENTS

GI 00124.00 Purpose of the Special Provision.

Special Provision 00124 – Green Infrastructure Submittal Requirements is intended to summarize all submittals for procedures, materials, and testing requirements associated with green infrastructure (GI) projects. If a Special Provisions for Green Infrastructure conflicts with this Section, then the Special Provision will take precedent. Unless otherwise specified, all submittals shall be provided to the City at least one month prior to delivery of materials to the project site.

GI 00124.01 GI Site Activity Plan Submittal.

- Description. The Site Activity Plan should be submitted and approved prior to commencement of any field activities and should contain a construction progress schedule, a Runoff Management Plan, a Site Access Plan, and a Site Utilization Plan.

- Construction Process Schedule: The Contractor shall submit Construction Progress Schedule as described in NO DPW Section 126.01. This schedule shall include the green infrastructure specific items listed in GI 00126.

- Runoff Management Plan for Construction of GI Facilities:

Method and plan outlining the Contractor’s approach to runoff management and the protection of green infrastructure facilities prior to, during, and following construction per GI 00126.

Plan to address sedimentation of GI facility per GI 00126.

- Site Access Plan

- Plan sheet and notes illustrating the Contractor’s approach to Site Access for construction related access, non-construction related access, and maintenance of traffic in and around the site per GI 00126.

Map showing haul roads for heavy and large equipment access per GI 00126. Haul roads that must be located within the footprint of the project shall also include a plan for de-compaction of the site.

Description and/or map of material delivery procedures per GI 00126. Storage and handling of materials shall be in accordance with specific sections in Specifications.

- Site Utilization Plan: Plan sheet and notes illustrating the Contractor’s approach to protecting green infrastructure facilities from construction traffic, material storage, and erosion/sedimentation caused by typical construction activity, including:

- A description of the open excavation protection that will be employed per GI 00126.

A description of how private property will be protected per GI 00126.

A description of how trees will be protected and a map showing trees to receive protection per GI 00126.

A description of how existing structures and utilities will be protected per GI 00126.

A description of how flow in sewers, drains, and water courses will be maintained during the progress of the work per GI 00126.

A map showing the location of anticipated material storage, laydown areas, and temporary facilities per GI 00126.

A description of how stored materials will be protected during the course of construction per GI 00126.

A description of methods for disposal of materials per GI 00126.

A description of the equipment and methods used to backfill with specified materials in a manner that does not compact the subgrade or backfill materials of green infrastructure facilities per GI 00126.

GI 00124.02 GI Earthwork Submittals.

At least one month prior to beginning excavation activities, submit a description of equipment and methods that will be used to excavate GI facility and backfill with specified materials in a manner that does not compact the subgrade per GI 00200.

GI 00124.03 Tree Protection Submittals.

Submittal requirements are contained in New Orleans Parks and Parkways Section 015639.

GI 00124.04 Bioretention Soil Submittals.

- Name, address, phone, and contact for each laboratory that will be used to test and/or certify the materials.

- Description of the equipment and methods that will be used to mix the aggregate, planting soil, and compost to produce the bioretention soil media.

- One month prior to installation of material submit bioretention soil mixture, mulch, sand, topsoil, and bed edging material sources, manufacturer’s literature, samples, product data, testing, and laboratory certifications per GI 01005.

- One month prior to installation of any materials the Contractor must submit documentation of the Contractor’s qualifications which are in accordance with GI 01005.

GI 00124.05 Aggregates Submittals.

- Sieve analysis and void content for all aggregates supplied for every 300 tons of material delivered per NO DPW Section C604.08.

- List of materials proposed for work under NO DPW Section 604.08 including name and address of materials producers and the locations from which the materials are to be obtained.

- Other aggregate submittals as required in GI 01003.

GI 00124.06 Pervious Paving Submittals.

- List of materials to be provided under the applicable pervious paving special provision including the name and address of the materials producers and the locations from which the materials are to be obtained:

- Certificates signed by the material producer and the contractor stating that the materials meet or exceed the specified requirements defined in the applicable pervious paver special provision.

- Manufacturer’s product data, color chart, and texture options indicating conformance with the project requirements.

- Pervious paving samples as defined in the applicable special provision.

- Test results from an independent testing laboratory for compliance of pervious paving requirements for resistance to freeze thaw and other applicable requirements defined in the applicable pervious pavement special provision.

- Contractor’s qualifications/credentials, including subcontractors, demonstrating competent work with the applicable pervious paver material and additional requirements as defined in the applicable pervious paver special provision.

- Construction procedures and quality control plan.

GI 00124.08 GI Establishment Submittals.

- Contractor qualifications.

- Certifications for new materials and plants that are installed during the Period of Establishment per NO DPW SP 00720.02.

- Material data, testing results, certifications and other submittal data that has been updated or revised since the date of initial submittals as required per NO DPW SP 00720.02.

- List of the anticipated establishment visit dates and total number of visits in accordance with the Applicable Standards defined in NO DPW SP 00126.

- Written maintenance schedule per NO DPW SP 00720.05.

- Monthly report of activities during the previous month describing the information required per NO DPW SP 00720.06.

GI 00124.09 GI Design Component Submittals.

- Description: This work consists of submittal requirements for the installation of components for the use in green infrastructure applications including tree grates, trench drains, wheel guards, cleanouts, utility sleeves, and weirs.

- Submittals: The Contractor shall prepare drawings and submittals when required by the Contract Documents and after verifying applicable field and plan elevations, dimensions, geometries, and condition.