GI 00204

EROSION AND SEDIMENTATION CONTROL FOR

GREEN INFRASTRUCTURE FACILITIES

GI 00204.01 Description.

Furnish all labor, materials, equipment and incidentals required and perform all installation, inspection, maintenance, removal and area cleanup related to erosion and sedimentation control work as shown on the Drawings and as specified herein. The work shall include, but not necessarily be limited to; installation of stabilized construction access, staging areas, silt fences, stone filter boxes, stone filter berms, sediment removal and disposal, device maintenance, removal of temporary devices, temporary mulching, erosion control matting installation, inspection, maintenance of installed devices, removal, and final cleanup.

The Contractor will comply will all applicable requirements of City of New Orleans Department of Public Works Specification C204, Environmental Protection and Stormwater Pollution Prevention Plan. This Special Provision is intended to be in addition to the Standard Specification.

GI 00204.02 Applicable Standards.

City of New Orleans Department of Public Works General Specifications for Street Paving (Current Standards)

GI 00204.03 Requirements.

All Projects which include a Green Infrastructure facility, no matter the size of the disturbed area, are required to:

- Prepare a Stormwater Pollution Prevention Plan (SWPPP). This plan will describe how the Contractor will control runoff from construction site, keeping all soil, dirt, and pollutants within the limits of the site, AND AWAY FROM ALL GREEN INFRASTRUCTURE FACILITIES.

Green Infrastructure facilities shall be treated as large stormwater inlets, special precautions must be taken to keep the facility from polluting, clogging, and compacting.

Green Infrastructure SWPPP shall include, beyond what is identified in NO DPW C204:

- Site description with location and type of Green Infrastructure identified

Controls to be used to reduce pollution and sediment from entering the GI facility

Installation methods of control measures

Schedule of GI control measure installation

Inspections and Record Keeping

- For projects with Green Infrastructure facilities after construction is completed and soil has been stabilized, fill out a Completion Report and submit it to DPW project manager.

GI 00204.04 Submittals.

Contractor shall submit to Designer the following information:

- Technical product literature for all commercial products to be used for erosion and sedimentation control shall be submitted within 10 working days after the award of the Contract.

Copies of the Contractor prepared SWPPP (Stormwater Pollution Prevention Plan) and DEQ Erosion and Sediment Control Permit if required must be submitted to the Designer prior to commencement of construction activities.

GI 00204.05 Quality Assurance.

Contractor shall be responsible for the timely installation and maintenance of all sedimentation control devices necessary to prevent the movement of pollutants and sediment into Green Infrastructure facilities via surface runoff or underground drainage systems. Measures in addition to those shown on the Drawings necessary to prevent the movement of pollutants and sediment into the Green Infrastructure facility shall be installed, maintained, removed, and cleaned up at the expense of the Contractor. No additional charges to the Owner will be considered.

Inspection: Contractor shall inspect and repair or replace, as needed, all project site Stormwater Control Measures (SMCs) a minimum of:

- Weekly, and

Before, during, and after a rain event of more than 1 inch in 24 hours.

GI 00204.06 Materials.

See NO DPW C204.02 for SWPPP and SCMs general information, construction practices, temporary erosion control methods, maintenance, and materials.

GI 00204.07 Protection During Suspension of Operations.

Prior to the suspension of operations, the Contractor shall shape the top of the earthwork in such manner as to permit runoff of rainwater and shall construct earth berms along the top edges of embankments to intercept runoff water. Temporary slope drains shall be provided in the earth berm to carry runoff. When such preventive measures fail, the Contractor shall immediately take other action as necessary to prevent erosion and siltation of the Green Infrastructure facility.

GI 00204.08 Removal and Final Cleanup

- Erosion control devices shall be maintained until all disturbed earth has been adequately stabilized, at which time it shall be removed at the direction of the Designer.

Any other erosion and sediment control devices used by the Contractor in addition to the specified contract items shall be removed at the direction of the Designer.

After erosion control devices are removed, dispose of silt and waste materials in proper manner. Regrade all areas disturbed during this process and stabilize against erosion with surfacing materials as indicated on the Drawings.

GI 00204.09 Measurement.

When temporary erosion and pollution control measures are required due to the Contractor's negligence or failure to install permanent controls, such work shall be performed by the Contractor at no direct pay. Required temporary erosion and pollution control work which is not due to the Contractor's negligence will be measured as follows:

- When separate items for temporary erosion control devices are included in the contract, and the work is ordered, the quantities to be paid for will be the area in square feet or acres of Temporary Seeding; the volume in cubic yards (CY) of Sandbagging with the measurement of sand being made in a batch box or other satisfactory means; the number of hay bales placed; the length in feet (L) of Temporary Slope Drains measured along the ground surface and Silt Fencing measured along ground surface between end posts; the number of Sediment Basins and Sediment Check Dams acceptably constructed; the cubic yard of aggregate for Construction Entrances.

Temporary erosion control items may be eliminated when conditions do not justify their use. When temporary erosion control work is ordered and is not covered by contract items, the work shall be performed as extra work in accordance with LaDOTD Subsection 109.04 except that no extra work order will be required prior to starting work.

The construction of temporary earth berms along edges of Green Infrastructure facilities to prevent erosion during grading and subsequent operations will not be measured for payment.

In case of the failure of the Contractor to control erosion, or siltation, the Designer may employ outside assistance to provide the necessary corrective measures, and the cost thereof will be deducted from payments for the work to the Contractor. Partial payments will be withheld until satisfactory temporary erosion control is established.

GI 00204.10 Payment.

Unless otherwise specified, no direct payment shall be made for obtaining and maintaining the necessary permits, preparation, installation, and inspection of the stormwater control measures including all labor equipment or materials required to complete the SWPPP.

Payment for temporary erosion control items that are included as contract items will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | PAY UNIT |
| --- | --- | --- |
| GI-0204-01 | TEMPORARY SANDBAGGING | CY |
| GI-0204-02 | TEMPORARY STRAW BALES | EA |
| GI-0204-03 | TEMPORARY SLOPE DRAINS - LINEAR | LF |
| GI-0204-04 | TEMPORARY SEDIMENT BASINS | EA |
| GI-0204-05 | TEMPORARY SEDIMENT CHECK DAMS | EA |
| GI-0204-06 | TEMPORARY SILT FENCING | LF |
| GI-0204-07 | TEMPORARY STONE CONSTRUCTION ENTRANCE | CY |
| GI-0204-08 | CURB INLET DRAIN FILTER (9” DIA. X 8’L) | EA |
| GI-0204-09 | CURB INLET DRAIN FILTER (6” DIA. X 6’L) | EA |
| GI-0204-10-A | TEMPORARY SEEDING (SPECIES) | SF |
| GI-0204-10-B | TEMPORARY SEEDING (SPECIES) | AC |