GI 00703

DRAIN INLETS

FOR GREEN INFRASTRUCTURE FACILITIES

- GI 00703.01 Description.

- This work shall include, but not be limited to, the installation and protection of all drain inlets shown on the Drawings as specified herein. This Special Provision is intended to be a compliment to technical specifications for specific products that are provided by the project Engineer/Landscape Architect.

GI 00703.02 Submittals.

Contractor shall submit to the Engineer/Landscape Architect the following information:

Shop Drawings for drain inlets: Include elevations, sections, details, frames, covers and grates.

GI 00703.03 Materials.

- Domed Drain Inlet: As indicated on the Drawings.

- Material: PVC, HDPE, Cast Iron, or Metal

Shape: Square or round

Color: To be determined by Project Engineer/Landscape Architect

Flow rate: To be determined by Project Engineer/Landscape Architect

- Flat Drain Inlet: Size to be determined by project Engineer/Landscape Architect. All flat drain inlets to be suited for specific load requirements expected, be ADA Compliant, and heel-proof when applicable.

Material: HDPE or Metal

Shape: Square or round

Color: To be determined by Project Engineer/Landscape Architect

Flow rate: To be determined by Project Engineer/Landscape Architect

GI 00703.04 Installation.

Excavation, trenching, and backfill are specified by the project Designer in GI 00200 – Earthwork for Green Infrastructure Projects.

- General locations and arrangements are indicated on drawing set. Contractor to follow piping manufacturer’s specifications when installing pipes. Set drain inlets to elevations indicated on drawings.

- Contractor to set all inlet and pipe elevations to ensure positive drainage is achieved.

- Drain inlets to be installed after all piping work is completed and verified by project Engineer/Landscape Architect.

- When installing flat drain inlets, set covers in surface material flush with pavement surface.

- When applicable, install aggregate around inlet, as per details. Aggregate should slope away from the pipe and complete surround the base of the stand pipe.

GI 00703.05 Quality Control.

- Inspect interior of pipe and inlet to determine whether line displacement or other damage has occurred. Notify project Engineer/Landscape Architect if any damage has been discovered.

- Leaks or other defects must be repaired, re-inspected, and confirmed by project Engineer/Landscape Architect. Replace leaking pipes and/or inlets with new materials.

- Protect all inlets from runoff entering them system until all earth work and exterior improvements are complete. This includes, but is not limited to, backfilling soil, grading, and installing plant materials.

GI 00703.06 Removal and Final Cleanup.

- During construction, Contractor to inspect each installed inlet after any weather event, and remove any debris.

GI 00703.07 Measurement.

- Drain inlets will be measure per each unit (EA). This will encompass all labor, materials, overhead, and profit to install and maintain through the warranty period.

GI 00703.08 Payment.

Payment for drain inlets will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | UNIT |
| --- | --- | --- |
| GI-0703-01 | Drain Inlets (Domed Drain Inlet) (Size) | Each |
| GI-0703-01-A | Drain Inlets (Domed Drain Inlet) (6”) | Each |
| GI-0703-01-B | Drain Inlets (Domed Drain Inlet) (12”) | Each |
| GI-0703-02 | Drain Inlets (Flat Drain Inlet) (Size) | Each |
| GI-0703-02-A | Drain Inlets for GI (Flat) (12”) | Each |
| GI-0703-02-B | Drain Inlets for GI (Flat) (18”) | Each |
| GI-0703-02-C | Drain Inlets for GI (Flat) (24”) | Each |