GI 00704

UNDERDRAINS FOR GREEN INFRASTUCTURE FACILITIES

GI 00704.01 Description.

- Furnish all labor, materials, tools and equipment necessary for furnishing and installing underdrain as shown on the Drawings, Standard Details, and as specified herein.

GI 00704.02 Materials.

- The type and size of perforated underdrain pipe shall be as specified on the Drawings.

- Slotted or perforated and non-perforated polyvinyl chloride (PVC) plastic pipe shall conform to the requirements of ASTM D 3034.

- Slotted or perforated high density polyethylene (HDPE) corrugated pipe shall conform to the requirements of ASTM F667.

- Slotted pipe shall have at least two rows of slots cut perpendicular to the axis of the pipe or at right angles to the pitch of corrugations and with the centerlines of the rows separated by one-third the circumference of the pipe. Slots shall have a width between one-sixteen inch and one-tenth inch and shall have a length, as measured along the inside circumference, of one inch to one and one-fourth- inch. Spacing of the slots shall be between three-fourth-inch and one and one-half-inch along the axis of the pipe. Slots shall be formed in such a way that inflow of water through the slots will not be impeded by excessive residual material from the slotting procedure.

- Aggregates shall be in accordance with GI 01003 and material shall be placed to the dimensions as shown on the Drawings or Standard Details.

GI 00704.03 Installation.

- Trenches shall be excavated to the dimensions and grade shown on the Drawings or as directed by the Designer. A nominal four-inch layer of granular backfill material shall be placed in the bottom of the trench for its full width and length.

- Geotextile, if called for on the Drawings, shall be placed as shown on the Drawings and the Standard Details. Geotextile shall be in accordance with GI 00206.

- Perforated pipe shall be placed with the perforations down. The pipe sections shall be joined securely with the appropriate coupling bands or fittings.

- After the pipe installation has been inspected and approved, granular backfill material shall be placed.

GI 00704.04 Measurement.

- Underdrain pipe will be measured in linear feet and will include all costs for furnishing all materials, labor, tools, equipment and incidentals (including pipe risers, caps, and fittings) to complete the work.

GI 00704.05 Payment.

- Payment for underdrain pipe will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | PAY UNIT |
| --- | --- | --- |
| GI-0704-02 | Underdrain Pipe – Perforated – PVC (Size) | LF |
| GI-0704-02-A | Underdrain Pipe – Perforated – PVC (4”) | LF |
| GI-0704-02-B | Underdrain Pipe – Perforated – PVC (6”) | LF |
| GI-0704-02-C | Underdrain Pipe – Perforated – PVC (8”) | LF |
| GI-0704-02-D | Underdrain Pipe – Perforated – PVC (10”) | LF |
| GI-0704-02-E | Underdrain Pipe – Perforated – PVC (12”) | LF |
| GI-0704-03 | Underdrain Pipe – Solid – PVC (Size) | LF |
| GI-0704-03-A | Underdrain Pipe – Solid – PVC (4”) | LF |
| GI-0704-03-B | Underdrain Pipe – Solid – PVC (6”) | LF |
| GI-0704-03-C | Underdrain Pipe – Solid – PVC (8”) | LF |
| GI-0704-03-D | Underdrain Pipe – Solid – PVC (10”) | LF |
| GI-0704-03-E | Underdrain Pipe – Solid – PVC (12”) | LF |
| GI-0704-04 | Cleanout – PVC (Size) | EA |
| GI-0704-04-B | Cleanout – PVC (6”) | EA |
| GI-0704-05 | Underdrain Pipe – Perforated – HDPE (Size) | LF |
| GI-0704-05-A | Underdrain Pipe – Perforated – HDPE (4”) | LF |
| GI-0704-05-B | Underdrain Pipe – Perforated – HDPE (6”) | LF |
| GI-0704-05-C | Underdrain Pipe – Perforated – HDPE (8”) | LF |
| GI-0704-05-D | Underdrain Pipe – Perforated – HDPE (10”) | LF |
| GI-0704-05-E | Underdrain Pipe – Perforated – HDPE (12”) | LF |