GI 00708

MODULAR SUSPENDED PAVEMENT SYSTEMS

FOR GREEN INFRASTRUCTURE FACILITIES

- GI 00708.01 Description.

- This work shall include, but not be limited to, the installation and protection of all Modular Suspended Pavement Systems and plant material shown on the Drawings and as specified herein. This Special Provision is intended to be a complimentary to technical specifications for specific products that are provided by the project Landscape Architect.

- GI 00708.02 Requirements.

- Installation of plant material in the public right-of-way should follow the City of New Orleans Parks and Parkways standard Specification 32 90 00 – Planting, as well as the requirements outlined in the New Orleans CZO – Article 23.

GI 00708.03 Submittals.

Contractor shall submit to the Landscape Architect the following information:

- Submit manufacturer’s product technical data.

- Construct a complete mock up in the presence of the Landscape Architect.

- Mock up shall include 50 square feet and include subbase compaction, drainage installation, aggregate base course, geotextiles, bioretention soil, and all other materials required by the Manufacturer.

- Compaction testing results: Submit results of all compaction testing required by the Manufacturer’s specifications to the Landscape Architect for approval.

- Product Certificates: For each type of manufactured product, from manufacturer, and complying with the following:

  - Manufacturer’s certified analysis for standard products.

  - Manufacturer’s letter of review and approval of the project, plans, details, and specification for compliance with product installation and requirements.

GI 00708.04 Quality Assurance.

Modular Suspended Pavement System and related products shall be installed by a qualified installer whose work has resulted in successful installation of bioretention soil and associated drainage systems, including vault structures.

- Submit list of three completed projects of similar scope and scale to the Owner, demonstrating capabilities and experience.

- Installer is required to maintain an experienced full time supervisor on Project work site when work is in progress.

GI 00708.05 Materials.

Deliver all materials in original, unopened containers, showing name and address of manufacturer. Protect materials from deterioration during delivery and while on Project site.

- Provide erosion-control measures to prevent erosion or displacement of bulk materials including, but not limited to, bioretention soil and aggregates.

- Store Modular Suspended Pavement System is a way that it is protected from sunlight. Modular Suspended Pavement System should be stored on a surface that is free from dirt, mud, or other debris.

- Handing of the Pavement System shall be done in a way that is appropriate for the site conditions. Equipment may include hands and forklifts as needed.

GI 00708.06 Execution.

- Layout Approval: Prior to the start of work, layout and stake the limits of excavation and horizontal and vertical control points sufficient to install the Modular Suspended Pavement Systems and required drainage features.

- Excavation: Excavate to the depths and shapes indicated on the drawings. Base of excavation shall be smooth soil, level and free of lumps and debris.

  - Confirm that the depth of the excavation is accurate to accommodate the depths and thickness of materials required throughout the extent of the excavation.

- Subgrade compaction: Check compaction of the subgrade below the Modular Suspended Pavement System and confirm that the subgrade soil is compacted to a minimum required by the Manufacturer’s specification in accordance with ASTM D 689.

- Geotextile Installation: Where indicated on the drawings, install geotextile over the compacted subgrade as per Manufacturer’s specifications.

- Perforated and Solid Drain Installation: Layout/stake the location of all drain lines as per Drawings. Adjust the alignments to confirm to the final locations of sleeves and risers.

- Aggregate Subbase Installation: Install aggregate subbase to the depths indicated on drawings and specified by Manufacturer, under the Modular Suspended Pavement System.

  - Compact aggregate subbase as per Manufacturers’ specifications. The grade and elevations of the aggregate shall be approved by the Landscape Architect /Engineer prior to proceeding with the installation of the Modular Suspended Pavement System.

- Modular Suspended Pavement System Installation: Identify the layout of the Modular Suspended Pavement System and the edges of paving around the tree planting area on the floor of the excavation.

  - Layout the first layer of Modular Suspended Pavement System frames on the prepared aggregate subbase. Frames shall be placed at a distance determined by the Manufacturer.

  - Anchor each frame into the subbase as per Manufacturer’s specifications.

  - Install layers of Modular Suspended Pavement System to achieve depth as indicated on the Drawings. Comply with Manufacturer’s requirements to correctly register and connect the frames together.

- Bioretention Soil Installation: Install Modular Suspended Pavement Systems to a level determined by Manufacturer’s specifications before beginning to install bioretention soil and backfill.

  - Compact the bioretention soil within the Modular Suspended Pavement System frames and backfill material outside the frames, in lifts as required by the Manufacturer.

  - Continue installation of Modular Suspended Pavement System frames as indicated on Drawings. Obtain final approval by the Landscape Architect of bioretention soil installation once it is complete.

    - Install the Modular Suspended Pavement System deck over each of the frames. Clean soil from the tops of the decks and secure them as per Manufacturer’s specifications.

    - Install and compact remaining backfill materials so that the soil outside of the frames is flush with the top of the installed deck.

- Paving above Modular Suspended Pavement System Installation: Place paving material over Modular Suspended Pavement System as specified in the Drawings.

- Tree Installation: Prior to planting trees, install additional bioretention soil as needed within the tree opening. Ensure that planting soil under the tree root ball is compacted to no less than 90% to prevent settling of the root ball.

  - Cover the planting soil with a minimum of three inches of pine straw mulch, as indicated on drawings. Mulch should be level with adjacent finished grade of paving.

GI 00708.07 Protection.

- Ensure that all construction traffic is kept away from the limits of the Modular Suspended Pavement System until the final surface materials are in place. No vehicles shall drive directly on the Modular Suspended Pavement System.

- When a vehicle must cross the Modular Suspended Pavement System that does not have the final paving surfaces installed, use construction mats designed to distribute vehicle loads to levels that would be expected at the deck surface once final paving has been installed.

GI 00708.08 Removal and Final Cleanup

- Perform clean up during the installation of work and upon completion of work. Maintain the site free of soil and sediment, free of trash and debris. Remove all excess soil materials, debris, and equipment from site. Repair any damage to adjacent materials and surfaces resulting from installation of this work.

GI 00708.09 Measurement.

- Modular Suspended Pavement Systems will be measured in square feet (SF). This will encompass all labor, materials, overhead, and profit to install and maintain through the warranty period. Necessary excavation and disposal of excess excavated materials will not be measured for payment in this Division.

GI 00708.10 Payment.

Payment for Modular Suspended Pavement Systems will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | PAY UNIT |
| --- | --- | --- |
| GI-0708-01 | Modular Suspended Pavement System (Depth) | SF |