GI 00720

ESTABLISHMENT OF

GREEN INFRASTRUCTURE FACILITIES

- GI 00720.01 Description.

- Furnish all labor, materials, equipment and incidentals required to full establish the plant material during the establishment period as listed in Section 00720.04, Warranty. The work shall include, but not be limited to, inspection, maintenance, replacement, mulching, and weeding.

GI 00720.02 Submittals.

Contractor shall submit to the Landscape Architect the following information:

- Product data for all:

- Bioretention Soil submittals per Specification Section GI 01005.

Any other products the Contractor proposes to use during the establishment period.

- All products shall be approved in writing by the Landscape Architect prior to use on the Project.

GI 00720.03 Quality Assurance.

Contractor shall be responsible for the timely installation and maintenance of all planted areas within the Green Infrastructure Facility to prevent weed growth, erosion, damage, death, or unsatisfactory growth of plant materials. All measures to maintain and establish the planting material of the Green Infrastructure Facility shall be at the expense of the Contractor. No additional charges to the Owner will be considered, except under extraneous circumstances that are beyond the Contractor’s control, i.e. acts of nature.

GI 00720.04 Warranty.

- Contractor agrees to repair or replace plantings and accessories that fail in materials, workmanship, or growth within specified warranty period.

- Warranty Periods: from date of Substantial Completion:

  - Trees, shrubs, vines, grasses, perennials, groundcovers, and other plants: 36 months

  - Turf grasses: 60 days

    - When initial maintenance period has not elapsed before end of planting season, or if turf grass is not fully established, continue maintenance during the next planting season.

    - Planting season is defined below:

      - Trees and Shrubs: November to March

      - Turf Grass: March to August

GI 00720.05 Execution.

- Maintain plantings by pruning, cultivating, watering, weeding, mulching, resetting to proper grades, and performing other operations as required to establish healthy, viable plant material.

- Fertilizer, herbicides, and/or pesticides are not permitted for use in any Green Infrastructure facility.

- Fill in, as necessary, soil subsidence that may occur because of settling or other processes. Replenished soil shall be the same mix as originally specified in the Project

- Protect plants from damage due to one site operations. Maintain protection during installation and maintenance periods. Treat, repair, or replace damaged plant materials.

- Twelve months after Substantial Completion is issued, remove tree stakes, guide wire, and watering bags from planted trees and shrubs, as required to keep plant material in good form and health.

- Provide Owner with a detailed Maintenance Manual, specific to Project site, at Final Acceptance.

  - Manual should include recommended procedures by the Owner for maintenance of plants during a calendar year.

GI 00720.06 Establishment Service.

- Initial Establishment Service: Provide establishment service by skilled employees of landscape contractor. Execute as required in SP 00720.05. Begin establishment services immediately after plants are installed and continue until plant material is acceptably healthy and well established, but not less than the specific establishment periods, as defined in SP 00720.04, Warranty.

GI 00720.07 Measurement.

- Trees will be measured per each tree. This will encompass all labor, materials, overhead, and profit to install and maintain through the warranty period.

- Bioretention Plantings will be measured per each plant. This will encompass all labor, materials, overhead, and profit, to install and maintain through the warranty period.

- Pine straw mulch will be measured per square yard (SY). This will encompass all labor, materials, overhead, and profit to install and maintain through the warranty period.

- Turf maintenance will be measured per square yard (SY). This will encompass all labor, materials, overhead, and profit to install and maintain through the warranty period.

GI 00720.08 Payment.

Payment for Green Infrastructure Establishment will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | PAY UNIT |
| --- | --- | --- |
| GI-0720-01-A | Turf Grass (Bermuda) (Hydroseed) | SY |
| GI-0720-01-B | Turf Grass (Bermuda) (Sprigged) | SY |
| GI-0720-02 | Tree (Species, (caliper or gallon)) | EA |
| GI-0720-02-A | Tree (Creole Queen Elm) | EA |
| GI-0720-02-B | Tree (Overcup Oak) | EA |
| GI-0720-02-C | Tree (Pond Cypress.) | EA |
| GI-0720-02-D | Tree (Creole Queen Elm) | EA |
| GI-0720-02-E | Tree (Swamp Red Maple) | EA |
| GI-0720-02-E | Tree (Taxodium distichum – Bald Cypress) | EA |
| GI-0720-02-F | Tree (Magnolia virginiana – Sweetbay Magnolia) | EA |
| GI-0720-02-G | Tree (Nyssa sylvatica – Black Gum) | EA |
| GI-0720-03 | Bioretention Plant (Species, (gallon)) | EA |
| GI-0720-03-A | Bioretention Plant (Juncus x Blue Dart’ - Blue Dart Rush, 1 Gal) | EA |
| GI-0720-03-B | Bioretention Plant (Dietes vegeta - African Iris, 3 Gal) | EA |
| GI-0720-03-C | Bioretention Plant (Hymenocallis liriosme - Spider Lily, 3 Gal) | EA |
| GI-0720-03-D | Bioretention Plant (Muhlenbergia capillaris - Pink Muhly, 3 Gal) | EA |
| GI-0720-03-E | Bioretention Plant (Sabal minor - Dwarf Palmetto, 3 Gal) | EA |
| GI-0720-04 | Pine Straw Mulch | SY |
| GI-0720-05 | Hardwood Mulch | SY |
| GI-0720-06 | Pine Nugget Mulch | SY |