GI 01005

BIORETENTION SOIL

FOR GREEN INFRASTRUCTURE FACILITIES

- GI 01005.01 Description.

The work shall include, but not be limited to, the installation, performance, and protection of all specified soil mixes as shown on the Drawings and as specified herein. This Special Provision is intended to be complimentary to technical specifications provided by project Landscape Architect.

GI 01005.02 Submittals.

Contractor shall submit to the Landscape Architect the following information:

Samples for each bulk-supplied material in sealed containers labeled with content, source, and date obtained; providing an accurate representation of composition, color, and texture.

GI 01005.03 Quality Assurance.

Contractor shall be responsible for an independent laboratory; experienced in soil science, soil testing, and plant nutrition; with the experience and capability to conduct the testing indicated, and that specializes in types of tests to be performed.

GI 01005.04 Products.

- Bioretention Soil: manufactured from off-site sources. Verify suitability of soil media to produce viable planting soil and provide desired infiltration rate by testing and amending in accordance with soil report and upon approval from the Landscape Architect.

- Bioretention Soil shall provide for an infiltration rate that is dependent on the variables from the Stormwater Calculator. Infiltration rates of the bioretention soil will vary based upon site specific conditions. Sample of the Bioretention Soil will be submitted to the Landscape Architect and verified to meet infiltration requirements.

- Submit sample of Bioretention Soil to Landscape Architect a minimum of fourteen (14) days before scheduled installation. Installation of Bioretention Soil is not to be performed without the written acceptance of the soil sample by the Landscape Architect.

GI 01005.05 Execution.

- Verify that no foreign or deleterious material or liquid such as paint, paint washout, concrete slurry, concrete layers or chunks, cement, plaster, oils, gasoline, diesel fuel, paint thinner, turpentine, tar, roofing compound, or acid has been deposited in Bioretention Soil.

- Placing manufactured Bioretention Soil over non-woven geotextile fabric.

- Install Bioretention Soil on site in its final, blended condition, do not apply materials or till if existing soil or subgrade is frozen, muddy, or excessively wet.

- Subgrade Preparation: Till subgrade to a minimum depth of six (6) inches.

- Application: Spread Bioretention Soil to a total depth as indicated on Drawings, but not less than required to meet finish grades after natural settlement. Do not spread if soil or subgrade is frozen, muddy, or excessively wet.

  - Lifts: Apply Bioretention Soil in lifts not exceeding more than four (4) inches in loose depth for material compacted by hand-operated tampers.

- Compaction: Do not compact Bioretention Soil after it is spread unless indicated on Drawings.

- Finish Grading: Grade Bioretention Soil to a smooth, uniform surface plane with loose, uniformly fine texture. Roll and rake, remove ridges, and fill depressions to meet finished grades.

- Before planting, obtain Landscape Architect’s acceptance of finished grading. Restore areas if eroded or otherwise disturbed after finish grading.

GI 01005.06 Field Quality Control.

- Infiltration Testing: For each Green Infrastructure Facility containing Bioretention Soils, infiltration tests will be performed. Perform test according to ASTM D3385-09.

- Soil will be considered defective is it does not pass ASTM D3385-09 and meet infiltration rates required in Section 01005.04 – (a) (1).

- Label each sample and test report with the date, location keyed to a site plan or other location system, visual condition when and where the test was performed, and testing depth.

- Protection Zone: Protect areas of Bioretention Soil from compaction, disturbance, and contamination. In order to prevent compaction, disturbance, and contamination of Bioretention Soil, the following practices are prohibited from backfilled areas:

- Storage of construction materials, debris, or excavated material;

- Parking vehicles or equipment;

- Vehicle traffic;

- Foot traffic;

- Erection of sheds or structures;

- Impoundment of water;

- Excavation or other digging unless otherwise indicated.

GI 01005.07 Measurement.

- Bioretention Soil will be measured by the cubic yard (CY). The length and width will be measured to the points established by the plans at a minimum depth of eight (8) inches, unless otherwise noted on the plans. Necessary excavation and disposal of excess excavated materials will not be measured for payment in this Division.

GI 01005.08 Payment.

Payment for Bioretention Soil will be made at the contract unit prices. Payment will be made under:

| ITEM NO. | PAY ITEM | PAY UNIT |
| --- | --- | --- |
| GI-1005-01 | Bioretention Soil | CY |